_ __    _    __   _    ,

)  ) _//   /  ) ' )  /    Amiga module player

M O D  /--'  /    /--/   /  /     for a 10Mhz or faster

/     /___ /  (_  (__/_     IBM compatible PC

//

Version 1.10     (/        (C) Mark J Cox 1991

____________________________________________________________________________

Contents:

o What are modules

o How to use MODPLAY

o How to get modules & contact author

and  o How to make a D/A converter for 5 pounds

that works with MODPLAY, PLAY and other software

____________________________________________________________________________

Introduction

------------

Modplay allows you to play (most) Amiga soundtracker/noisetracker files

on a PC (10Mhz or faster clock speed).  CGA minimum is required for the

graphical display whilst playing, but no display at all is required

just to play modules.  MODPLAY detects automatically if a module is

corrupt, and detects if it is a 15 or 30 instrument module.

What are Modules?

-----------------

A module consists of 15 (or 30) sampled instruments and a four channel

music track.  By playing the instruments at different pitches a

soundtrack lasting many minutes can be produced.  The PC speaker is not

really up to handling modules, since in total we have a 10 bit sample

to be played, and the speaker can only handle 5 (with difficulty!).  A

D/A converter gives 8 and the stereo version gives effectively 9.

Implementation:

---------------

MODPLAY implements all Amiga effects such as volume sliding, vibrato etc.

except for the looping which, although implemented as described in

Soundtracker v3.0 (plays all the sample first, followed by the loop

the rest of the time) does not seem to work with some modules. (See bugs

section)

Instructions:

-------------

There are two ways of using MODPLAY - the first from the command line and the

second in 'interactive mode'

Command Line:

-------------

From the command line a module can be played through any of the output

devices with or without a moving graph display.  The format is

MODPLAY [/q] [/0..5] filename [.MOD]

Items in [ ] are optional.   The /q if present tells MODPLAY not to

display any text and so the module will play without disturbing the

text display.  (You could use this in a game's title screen written

in C or Basic, by doing a system or shell command - MODPLAY exits when

a key is pressed).  The /number tells MODPLAY what hardware to play

the module through

/0 is the PC Speaker

/1 is an D/A converter on printer port 1

/2 is                  on printer port 2

/3 is 2 D/A converters, one on port1 the other on port2 (gives stereo)

/4 is 2 D/A converters as above but giving mono.

/5 is a D/A converter on printer port 1 (thats on a video card)

/6 is a prototype board

These are explained in more detail later on.

Playing is stopped by pressing *any* key, or when the module is finished

(some modules loop around to the beggining again and so never finish)

Interactive:

------------

Typing MODPLAY /i will scan the current directory for all module file.

These are then displayed in a list.  The cursor keys, PgUp, PgDn, Home

and End allow you to select a module;  the 'c' key alters the destination

of the sample (like /0,/1.. above); the 's' key displays the samples that

make up the module; ESC key exits and pressing SPACE or RETURN will play

the highlighted module.

Source of Modules:

------------------

There are many sources, Including myself.  Modules usually come archived

in LZH form, these can be unpacked with the LHARC program which can be

found on most MSDOS BBS's.  The files will usually unpack as 'mod.something'

and MSDOS cuts them short to 'mod.SOM' and it is best to rename these to

'something.mod' so that MODPLAY will automatically detect them.

Speaker Routines:

-----------------

The PC Speaker routines I used in PLAY and RESPLAY were not good enough for

MODPLAY.  PLAY and RESPLAY worked by linearly assigning the 64 different

waves the speaker could produce to the 256 levels of sample.  Most of

MODPLAYs information is in the middle 30 or so levels, as its not very

often that all the four channels are at maximum volume together.  So I

have designed a new logarithmic table - this means that the sound will be

a little wierd from the speaker, but it is better than it was with the

PLAY algorithm.

Supported Hardware:

-------------------

MODPLAY will play modules through the PC Speaker as mentioned above, through

an D to A converter on any of the three possible parallel ports, in stereo

if you have an D to A converter on both LPT1 and LPT2 (this sounds really

good) and also through a prototype board (currently in development by

a friend - stereo replay and sampling).

How to make a D/A converter for five pounds:

--------------------------------------------

A simple circuit for HiFi quality sound needs only a few components (and

this circuit will work with lots and lots of other software)

You need:

IC1 a ZN426E (about 3 pounds)

C1 a 10micro-farad electrolytic capacitor (10V or greater)

C2 a 4.7micro-farad electrolytic capacitor (10V or greater)

R1 a 380ohm resistor (any sort)

CON1 a phono socket

CON2 a 25 way male D-type connector (for your parrallel port)

A bit of 10 way cable - or 10 short wires.

1 roll of double sided sticky tape (how'd that get there?)

A +5v supply

Connect the following pins of IC1 to the numbered pins of CON2 (should

be numbered on the socket).

IC1 - CON2      IC1 - CON2

3     9         12    5

2     8         11    4

1     7         10    3

13    6          9    2

Connect 0v of your power supply to pin 25 of CON2, the ground connection

of CON1, pin 7 of IC1 and also the -ve side of C2.

Connect +5v of your power supply to pin 14 IC1 and one end of the resistor

IC1 pin 5 and 6 are joined together and also to the +ve side of C2 and the

other end of the resistor.

IC1 pin 4 is connected to the +ve side of C1, the -ve side being connected

to the centre connection of the phono socket CON1.

Every pin except pin 8 of IC1 should now be connected somewhere. Connect

a +5v power supply, attach to a parallel port and plug in your amplifier

to the phono socket.

Filter?

-------

Usually you would put a filter, buffer and amplifier after the D/A

converter - but I have had no problems without and it sounds great to

me.

Getting a +5v power supply

--------------------------

Catalogues such as MAPLIN (uk) provide simple circuits for obtaining

a +5v power supply (it only has to be 100mA maximum).  Alternatively

you could tap +5v off the keyboard connector or use 3 1.5volt

batteries (or even 4 1.5volt batteries with two diodes in series to

drop the voltage).  In fact, anything that produces a voltage in the

range 4.5 - 5.5 volts.

Freeware:

---------

This software is Freeware - that means that there is no registration fee,

you can copy this software, give it to anyone, and use it for anything

(subject to the license at the end of this document).  If you like this

software, and would like the author to continue writing a gift would be

appreciated.  For gifts of 10 UK pounds or greater I will send a package

of the full circuit diagrams for sampling as well as the playback mentioned

here,  printed documentation for PLAY, RESPLAY and MODPLAY and two disks

with Samples and the latest versions of the programs.

If you want to save the hassle of downloading modules, I have (at present)

nearly 5Meg worth.  If you wish me to send you one, or more disks please

enclose a gift of:  2 pounds post and packing (non-UK 3 pounds) and then

1 pound for each High Density disk you require.  Or send an SAE for a list

of all current disks and their contents.

All disks branded, High Density 3.5" (1.4Meg)

Disk 1.     Latest versions of PLAY, RESPLAY and MODPLAY

Disk 2,3,4  Various Tracker modules

My address is at the end of this document. PS It took me 9 solid days

to write this, and 3 hours to document - I hope this makes you feel

guilty :-)

Credits:

--------

Many thanks must go to Mahoney & Kaktus for their Amiga assembler

noisetracker replay routine which provided much inspiration and

help with the module format and effects.

Bugs:

-----

Will not load modules that have samples greater than 64k.  I have found

that this only applies to about 5% of modules.  I wrote the routine to

be quick and at the time was told the maximum in a module was 64k, I later

found that that the maximum was 128k and this would mean a rewrite of two

essential routines.  I will get round to it eventually. The LOOP effect

seems to work on most modules - some modules it doesn't.  I'm looking into

this one.

_____________________________________________________________________________

Copyright/License/Warranty

--------------------------

This document and the program file MODPLAY.COM ("the software") are

copyrighted by the author.  The copyright owner hereby licenses

you to: use the software; make as many copies of the program

and documentation as you wish; give such copies to anyone; and

distribute the software and documentation via electronic means.

There is no charge for any of the above.

You are specifically prohibited from charging, or requesting

donations, for any such copies, however made; and from

distributing the software and/or documentation with commercial

products without prior permission.

NO FOR-PROFIT ORGANIZATION IS AUTHORIZED TO CHARGE ANY AMOUNT

FOR DISTRIBUTION OF COPIES OF THE SOFTWARE OR DOCUMENTATION,

OR TO INCLUDE COPIES OF THE SOFTWARE OR DOCUMENTATION WITH SALES

OF THEIR OWN PRODUCTS.

THIS INCLUDES A SPECIFIC PROHIBITION AGAINST FOR-PROFIT

ORGANIZATIONS DISTRIBUTING THE SOFTWARE, EITHER ALONE OR WITH

OTHER SOFTWARE, AND CHARGING A "HANDLING" OR "MATERIALS" FEE OR

ANY OTHER SUCH FEE FOR THE DISTRIBUTION.  NO FOR-PROFIT

ORGANIZATION IS AUTHORIZED TO INCLUDE THE SOFTWARE ON ANY MEDIA

FOR WHICH MONEY IS CHARGED WITHOUT PRIOR PERMISSION FROM THE

COPYRIGHT OWNER.

No copy of the software may be distributed or given away without

this document; and neither the program or document may be altered

in any way.

There is no warranty of any kind, and the copyright owner is not

liable for damages of any kind.  By using this free software,

you agree to this.

The software and documentation are:

Copyright (C) 1990/1991       Mark J Cox

All trademarks/registered names acknowledged.

_____________________________________________________________________________

Address:

--------

Mark J Cox                 Electronic Mail:

29 Lundie Close             (JANET) mjhc8@uk.ac.bradford.eleceng

Stenson Fields                      mjhc8@eleceng.bradford.ac.uk (non UK)

Derby                       (NETMAIL) Mark Cox  2:250/404.5

DE2 3AN                             (Kingdom of Greyhawk BBS)

(Or if before July 1991, 17 Kirkburn Place, Bradford, W Yorks BD7 2BZ

is quicker)

All comments on PLAY, RESPLAY, MODPLAY or future software is welcomed!